## Section 1: Basic Networking & IP Addressing

Learning Objectives:

- Understand LAN communication

- Configure static and DHCP IP addressing

- Perform subnetting and VLSM

- Troubleshoot basic connectivity issues

Recommended Tool:

- Packet Tracer (Primary)

- EVE-NG (Optional)

Scenarios (1–8):

• A small office has 3 PCs and 1 router.

Design a network so all PCs can communicate. Assign IPs manually.

• Two departments must be on different IP subnets but use the same switch.

How would you configure this?

• You are given the network 192.168.10.0/24.

Subnet it to support 4 departments, each needing 50 hosts.

• A PC cannot ping its default gateway.

Investigate and fix the issue.

• Configure DHCP on a router to serve IPs to 5 PCs.

• One PC must always receive the same IP address.

Implement a DHCP reservation.

• Explain and demonstrate the difference between:

- Static IP

- DHCP IP

• A network is running out of IP addresses.

Redesign it using VLSM.

Senario 1

Small office: 3 PCs + 1 router

Goal: All PCs communicate, manual IPs

✅ Possible? Yes — very basic LAN design.

Design

- Devices:

  - 1 Router

  - 1 Switch

  - 3 PCs

- Topology:
PCs → Switch → Router

IP Assignment (Example)

- Network: 192.168.1.0/24

- Router (LAN): 192.168.1.1

- PC1: 192.168.1.10

- PC2: 192.168.1.11

- PC3: 192.168.1.12

- Subnet Mask: 255.255.255.0

- Default Gateway (all PCs): 192.168.1.1

📌 Result: All PCs can ping each other and the router.

Scenario 2

Two departments, same switch, different subnets

✅ Possible? Yes — using VLANs.

Concept

- Same physical switch

- Logical separation using VLANs

- Router (or Layer 3 switch) routes between them

Example

- VLAN 10 → HR → 192.168.10.0/24

- VLAN 20 → Finance → 192.168.20.0/24

Configuration Summary

- Create VLANs on switch

- Assign ports to VLANs

- Configure router-on-a-stick (subinterfaces)

- Enable inter-VLAN routing

📌 Without routing → departments are isolated
📌 With routing → controlled communication

Scenario 3

Subnet 192.168.10.0/24 for 4 departments, 50 hosts each

✅ Possible? Yes — using subnetting.

Requirement

- 50 hosts → need 64 IPs

- Subnet mask: /26 (255.255.255.192)

Subnets

| Department | Network | Host Range |
| --- | --- | --- |
| Dept 1 | 192.168.10.0/26 | .1 – .62 |
| Dept 2 | 192.168.10.64/26 | .65 – .126 |
| Dept 3 | 192.168.10.128/26 | .129 – .190 |
| Dept 4 | 192.168.10.192/26 | .193 – .254 |

📌 Exactly fits 4 departments with room to spare.

Scenario 4

PC cannot ping its default gateway

✅ Possible? Yes — very common issue.

Troubleshooting Steps

- Check IP address (ipconfig)

- Verify default gateway

- Check subnet mask

- Ping local PC (127.0.0.1)

- Ping another PC

- Check cable / NIC

- Check router interface is UP

- Check firewall blocking ICMP

Typical Fix

- Wrong gateway

- Wrong subnet

- Interface shutdown

- Bad cable

📌 This is a classic exam + job interview question.

Scenario 5

Configure DHCP on a router for 5 PCs

✅ Possible? Yes — routers commonly act as DHCP servers.

Example (Cisco-style)

ip dhcp pool OFFICE

network 192.168.1.0 255.255.255.0

default-router 192.168.1.1

dns-server 8.8.8.8

Exclude router IP

ip dhcp excluded-address 192.168.1.1

📌 PCs set to DHCP will auto-receive IPs.

Scenario 6

One PC must always get the same IP (DHCP reservation)

✅ Possible? Yes — very common.

How It Works

- DHCP server binds:

  - MAC address → fixed IP

Example

ip dhcp pool PRINTER-PC

host 192.168.1.50 255.255.255.0

hardware-address 00AA.BBCC.DDEE

default-router 192.168.1.1

📌 Best for servers, printers, CCTV, critical PCs.

Scenario 7

Static IP vs DHCP IP (Explain & demonstrate)

✅ Possible? Yes — fundamental networking concept.

| Feature | Static IP | DHCP IP |
| --- | --- | --- |
| Assignment | Manual | Automatic |
| Changes | Fixed | Can change |
| Management | Admin-controlled | Centralized |
| Use Case | Servers, routers | PCs, phones |

Demonstration

- Static: Manually configure IP on PC

- DHCP: Set PC to “Obtain IP automatically”

📌 DHCP reduces errors, static improves predictability.